# WLAN Pi Wireless Engineering Platform

02 | Architecture

| Document Number | 02_Architecture |
| --- | --- |
| Version | 1.0.0 |
| Audience | Wireless engineers, maintainers, and contributors |
| Current Platform Version | 1.0.0 |
| Last Updated | 2026-08-21 |

Purpose — Define the platform components, boundaries, data flows, deployment relationships, and current vendor dependencies so the system can be understood and operated consistently across supported machines.

# 1. High-Level Architecture

The platform is organized around a Mac or Windows engineering workstation, a WLAN Pi Go used as the RF capture and wireless-engineering appliance, and supporting analysis applications. The source-controlled toolkit is maintained separately from operational artifacts such as captures, reports, and published documentation.

| Layer | Primary components | Role |
| --- | --- | --- |
| Engineer workstation | macOS or Windows; Git; SSH; toolkit source; MetaGeek App; Wireshark | Operator interface, deployment host, capture analysis, documentation and reporting |
| WLAN Pi Go | WLAN Pi OS; wlan0/wlanpi0; wlanpi-core; WebUI/nginx; Oscium services | RF survey, wireless profiling, OTA capture and local toolkit execution |
| Operational workspace | Captures; Reports; Survey_Exports; Releases; Docs; Deploy; Tools | Persistent engineering artifacts and distribution packages |
| Vendor dependencies | MetaGeek App; Oscium capture stack; Wireshark ExtCap | External capture and analysis integrations; some functions remain vendor-controlled |

ENGINEERING DATA PATH

ENGINEER WORKSTATION Toolkit source ¦ MetaGeek App ¦ Wireshark ¦ Git/SSH

USB / NETWORK MANAGEMENT PATH SSH / WebUI / TCP capture service as applicable

WLAN PI GO wlanpi-core ¦ wlan0 / wlanpi0 ¦ Oscium capture stack

OTA 802.11 ENVIRONMENT Beacons ¦ Probes ¦ Control ¦ Data ¦ RF metadata

ENGINEERING ARTIFACTS PCAPNG ¦ RF survey exports ¦ reports ¦ documentation

# 2. Hardware Components

The reference hardware is the WLAN Pi Go. The platform has been validated with the Go reachable at 198.18.42.1 over the USB management connection. The exact USB interface presented on the Mac may vary by host configuration; the validated reference Mac used interface en8 for USB Ethernet management.

| Component | Validated state | Notes |
| --- | --- | --- |
| WLAN Pi Go | Validated | Primary wireless engineering appliance. |
| USB-C management | Validated | Mac reaches Go at 198.18.42.1 in the reference setup. |
| Wi-Fi radio interfaces | Validated | wlan0 and wlanpi0 present on the reference Go. |
| OTA capture stack | Validated | oscium-capture service active; TCP 6174 listening. |

# 3. Software Components

| Component | Location / identity | Function |
| --- | --- | --- |
| Toolkit launcher | ~/wlanpi-toolkit/launcher.sh on Go | Menu-driven engineering workflows. |
| Toolkit modules | ~/wlanpi-toolkit/lib/ | Profiler, survey, analytics, dashboard, export and capture status modules. |
| WLAN Pi services | systemd services on Go | Core WebUI and system/runtime functions. |
| Oscium capture | oscium-capture.service; TCP 6174 | Capture service used by supported capture clients. |
| MetaGeek App | Engineer workstation | Validated OTA capture workflow and capture export path. |
| Wireshark | Engineer workstation | Deep packet analysis and offline PCAPNG analysis. Live Oscium ExtCap is currently vendor-pending. |

# 4. Platform Directory Layout

The workspace root is maintained separately from the Git source tree. The current reference workspace is ~/Documents/WLAN-Pi-Go-Toolkit on macOS.

| Directory | Purpose |
| --- | --- |
| Toolkit_Source/ | Git-controlled source code, launcher, modules, profiles, tests and deployment scripts. |
| Docs/Knowledge_Base/ | Published engineering documentation in Word format. |
| Captures/ | Operational packet-capture artifacts. |
| Reports/ | Generated engineering reports. |
| Survey_Exports/ | RF survey and survey-derived exports. |
| Releases/ | Release packages. |
| Deploy/ | Deployment-related artifacts. |
| Downloads/ | External tools or downloaded packages retained by the project. |
| Tools/ | Supporting binaries and utility packages, including the Oscium ARM64 binary and RockstarWiFi assets. |

# 5. Data Flow

The normal engineering flow separates collection from analysis. Wireless data is collected on the Go, passed to a supported analysis client or exported as a capture artifact, then reviewed in the appropriate engineering application.

### RF survey flow

WLAN Pi Go radio -> survey engine -> survey snapshot/package -> channel/security/neighbor/enterprise analyses -> exported reports.

### OTA capture flow

WLAN Pi Go radio -> Oscium capture service -> supported capture client -> Radiotap PCAPNG -> Wireshark for frame-level analysis.

### Toolkit deployment flow

Engineer workstation -> SSH/deployment scripts -> /home/wlanpi/wlanpi-toolkit -> remote launcher and modules.

### Documentation flow

Validated procedure -> Word Knowledge Base document -> Docs/Knowledge_Base -> future release/package publication.

# 6. OTA Capture Architecture

The validated OTA workflow currently uses the MetaGeek App to access the WLAN Pi Go capture service and produce a PCAPNG file for analysis in Wireshark. The MetaGeek App has been observed to receive live RF data and the resulting PCAPNG has been verified as an IEEE 802.11 Radiotap capture with management, control, and data traffic.

| Stage | Component | State | Evidence / note |
| --- | --- | --- | --- |
| 1 | WLAN Pi Go RF capture | PASS | Oscium capture service active; TCP 6174 listening. |
| 2 | MetaGeek App | PASS | Live OTA data observed; APs, clients, channels and RF metrics displayed. |
| 3 | PCAPNG export | PASS | Exported capture verified in Wireshark. |
| 4 | Radiotap / 802.11 analysis | PASS | Radiotap headers and 802.11 management, control and data frames present. |
| 5 | Live Wireshark ExtCap | PENDING | MetaGeek reproduced the issue and escalated it to engineering. |

# 7. Deployment Architecture

The reference deployment process is workstation-driven. The source repository remains on the engineer workstation; deploy.sh copies the toolkit to /home/wlanpi/wlanpi-toolkit on the Go and then verifies the deployment.

Start working session and validate the Go runtime.

Use deploy.sh from Toolkit_Source.

Verify the remote directory and required modules.

Verify launcher syntax and menu rendering.

Run the requested workflow from the launcher.

# 8. Engineering Workflow Boundaries

| Workflow | Primary tool | Boundary |
| --- | --- | --- |
| RF survey and RF inventory | WLAN Pi Toolkit | Collection, classification, channel/security inventory and exports. |
| Client profiling | WLAN Pi Toolkit + Profiler | Association capability and client-side validation workflows. |
| OTA packet analysis | MetaGeek App + Wireshark | MetaGeek is the current validated capture path; Wireshark is the frame-analysis tool. |
| Documentation | Word Knowledge Base | Authoritative operational procedures and project knowledge. |

# 9. Known Vendor Dependencies

The platform intentionally keeps vendor-controlled components outside the core toolkit logic. This limits coupling and makes it possible to continue engineering work when a vendor integration is temporarily unavailable.

| Dependency | Current state | Platform impact |
| --- | --- | --- |
| MetaGeek App | Validated OTA capture client | Required for the current supported live OTA capture workflow. |
| Oscium ARM64 ExtCap | Vendor engineering pending | Live Wireshark capture remains unavailable until the compatibility issue is resolved. |
| Wireshark | Validated for offline PCAPNG analysis | Used for frame-level investigation of exported captures. |

# 10. Future Architecture

The architecture is intended to support additional engineering workflows without changing the core separation between collection, analysis, documentation, and operational artifacts. Future work is expected to extend capture validation, reporting, onboarding, and cross-platform deployment while preserving the existing repository and deployment boundaries.