WLAN Pi Wireless Engineering Platform

# 09 - OTA Capture Workflow

Operational procedure for validated over-the-air Wi-Fi packet capture

| Document Number | 09 |
| --- | --- |
| Audience | Wireless engineers, Wi-Fi support engineers, platform maintainers |
| Primary Capture Path | WLAN Pi Go -> MetaGeek App -> PCAPNG -> Wireshark |
| Current ExtCap Status | Live Wireshark ExtCap is vendor-engineering pending |

# 1. Purpose

This procedure defines the validated workflow for capturing over-the-air (OTA) IEEE 802.11 traffic with a WLAN Pi Go, using the MetaGeek App as the currently validated packet-capture client and Wireshark for post-capture analysis.

# 2. Scope

Covers connection and readiness validation for the WLAN Pi Go.

Covers starting an OTA packet capture through the MetaGeek App.

Covers exporting and opening the resulting PCAPNG file in Wireshark.

Covers capture-quality validation for engineering use.

Documents the current live Wireshark ExtCap limitation and the approved workaround.

# 3. Prerequisites

WLAN Pi Go powered and reachable from the workstation.

USB-C network connectivity to the Go, using the validated workstation setup.

WLAN Pi Go preflight completed; web UI and core services healthy.

MetaGeek App installed and able to authenticate.

Wireshark installed for PCAPNG analysis.

A suitable capture location and test objective defined before starting the session.

Important operational rule Do not treat an Ethernet capture of the Mac-to-Go management interface as an OTA Wi-Fi capture. The engineering capture must contain 802.11 frames with Radiotap metadata.

# 4. Validated Capture Architecture

| Stage | Component | Function |
| --- | --- | --- |
| 1 | WLAN Pi Go | Captures OTA 802.11 traffic and exposes the capture service. |
| 2 | MetaGeek App | Discovers the Go and receives/processes packet-capture traffic. |
| 3 | PCAPNG output | Preserves the captured 802.11/Radiotap frames for offline analysis. |
| 4 | Wireshark | Provides frame-by-frame protocol/RF analysis of the PCAPNG file. |

# 5. Start-of-Session Validation

1. Connect the workstation to the WLAN Pi Go using the validated USB-C connection.

2. Run the standard start-of-session preflight script: start-go-session.sh.

3. Confirm root filesystem is mounted read-write when required by the session workflow.

4. Confirm wlanpi-core.service, Web UI, and nginx are active.

5. Confirm the expected management ports are listening.

6. Confirm Wi-Fi interfaces are present on the Go.

7. Confirm oscium-capture.service is active when using the validated OTA capture path.

8. Do not begin packet capture if the preflight reports a failed service or missing dependency; troubleshoot the failed prerequisite first.

# 6. Start an OTA Capture in MetaGeek App

1. Launch MetaGeek App and authenticate with the working account.

2. Connect the WLAN Pi Go to the workstation.

3. Allow the application to discover the Go as a packet capture source.

4. Confirm the application no longer reports "No packet capture source."

5. Confirm live RF/packet-derived data begins to populate. A validated session showed networks, access points, clients, signal values, channel information, and client counts.

6. Define the capture objective before recording, such as general RF observation, management-frame analysis, roaming investigation, authentication testing, or client troubleshooting.

7. Start the capture and allow enough time to collect the traffic required for the specific objective.

# 7. What a Valid OTA Capture Looks Like

802.11 frames are decoded in Wireshark rather than Ethernet-only traffic.

Radiotap headers are present.

RSSI information is available on captured frames.

Channel information is available.

Management traffic such as Beacon, Probe Request, and Probe Response frames can be observed.

Control traffic such as ACK, CTS, or Block ACK may be observed when present in the RF environment.

Data and QoS traffic are visible when nearby stations generate data traffic.

# 8. Capture Validation Procedure

After exporting the PCAPNG, open it in Wireshark and verify that the file represents OTA 802.11 traffic. The following checks were validated against a real WLAN Pi Go capture.

| Component | Status | Validated Result |
| --- | --- | --- |
| File format | PASS | PCAPNG file opens successfully. |
| Encapsulation / Radiotap | PASS | Frames contain Radiotap and 802.11 radio information. |
| Beacon frames | PASS | Multiple Beacon frames observed. |
| Probe Requests | PASS | Client Probe Requests observed. |
| Probe Responses | PASS | AP Probe Responses observed. |
| Control frames | PASS | ACK, CTS, and Block ACK observed in the validated capture. |
| Data frames | PASS | Data, QoS Data, QoS Null, and Null traffic observed. |
| RSSI metadata | PASS | Per-frame RSSI values present. |
| Channel metadata | PASS | Channel values present. |
| PHY duration / rate | PASS | PHY duration and data-rate fields present in the validated capture. |

# 9. Recommended Wireshark Filters

| Purpose | Display Filter |
| --- | --- |
| All management frames | wlan.fc.type == 0 |
| All control frames | wlan.fc.type == 1 |
| All data frames | wlan.fc.type == 2 |
| Beacons | wlan.fc.type_subtype == 0x08 |
| Probe Requests | wlan.fc.type_subtype == 0x04 |
| Probe Responses | wlan.fc.type_subtype == 0x05 |
| EAPOL | eapol |

# 10. Authentication / Association Test

The validated general-purpose capture contained management, control, and data traffic, but did not intentionally generate a complete client authentication sequence. To validate that workflow, run a separate controlled capture while a test client joins a WLAN.

1. Start a fresh OTA capture.

2. Perform a controlled client join to the test WLAN.

3. Stop the capture after the authentication/association exchange is complete.

4. Check for Authentication and Association Request/Response frames.

5. For WPA/WPA2/WPA3 scenarios where applicable, check for EAPOL traffic.

6. Do not interpret the absence of those frames in a general RF capture as a capture failure; they may simply not have occurred during the capture window.

# 11. Export and Archive

Save the PCAPNG with a meaningful session name and timestamp.

Store OTA captures under the project Captures area using a consistent session naming convention.

Record the capture objective, location, channel/band focus, duration, and any relevant test-client details in the associated session notes or report.

Keep the original capture file immutable once it has been used as evidence for an incident or engineering analysis.

# 12. Live Wireshark ExtCap Status

Current limitation Live Wireshark capture through the osciumdump ARM64 ExtCap is not currently the approved primary workflow. Wireshark 4.6.6 hangs during external-capture-plugin initialization, and vendor support reproduced the issue. The vendor has escalated the finding to engineering.

The Go-side capture service is healthy.

The ARM64 osciumdump binary runs directly and the Go can be queried from the Mac.

The current workaround is MetaGeek App capture followed by PCAPNG analysis in Wireshark.

Do not fabricate or label a MetaGeek App capture as a live Wireshark ExtCap session.

# 13. Capture Failure Decision Tree

MetaGeek App shows no packet capture source -> verify the Go is connected, services are healthy, and the application is authenticated.

MetaGeek App shows a source but no RF data -> verify the Go capture service and test at a location with observable Wi-Fi traffic.

PCAPNG opens but contains Ethernet-only traffic -> the wrong interface/source was captured; do not use the file as OTA evidence.

PCAPNG contains Radiotap but expected frame classes are absent -> check whether the traffic was actually generated during the capture window before declaring capture failure.

Wireshark hangs while initializing the osciumdump ExtCap -> treat as the known vendor issue and use the MetaGeek App / PCAPNG workflow.

# 14. Acceptance Criteria

| Component | Status | Validated Result |
| --- | --- | --- |
| WLAN Pi Go ready | PASS required | Preflight is clean and capture service is healthy. |
| Capture source detected | PASS required | MetaGeek App identifies a packet capture source. |
| OTA traffic visible | PASS required | Live application data and/or PCAPNG shows wireless traffic. |
| Radiotap present | PASS required | Wireshark decodes Radiotap and 802.11 radio metadata. |
| Engineering-quality capture | PASS required | Management/control/data and RF metadata are available as applicable to the objective. |
| Wireshark ExtCap | Vendor pending | Use MetaGeek App workaround until engineering provides a compatible build/fix. |