# Capture Validation

WLAN Pi Wireless Engineering Platform

| Document | 12 – Capture Validation |
| --- | --- |
| Version | 1.0.0 |
| Purpose | Define a repeatable engineering gate for determining whether an OTA packet capture is usable for Wi-Fi troubleshooting. |
| Audience | Wireless engineers, network engineers, support engineers, and anyone operating the WLAN Pi platform. |
| Validation basis | Validated WLAN Pi Go + MetaGeek App + PCAPNG + Wireshark workflow. |
| Status | Production workflow validated on macOS; Windows procedure documented but not physically validated. |
| Known external issue | Live Wireshark Oscium ExtCap integration remains with MetaGeek engineering. |

# 1. Validation Philosophy

A capture is not considered engineering-ready merely because a PCAPNG file exists. Validation must confirm that the file contains native 802.11 frames with sufficient radio metadata to support the intended analysis. The validation process separates file-format integrity, 802.11 frame coverage, radio metadata quality, and scenario-specific evidence.

# 2. Minimum Acceptance Gate

Capture opens successfully in Wireshark without corruption warnings.

Encapsulation is IEEE 802.11 with Radiotap metadata (the validated workflow uses Radiotap link-layer data).

802.11 management, control, and data traffic can be identified when present in the capture.

RSSI information is present on captured frames.

Channel information is present and decodes correctly.

The capture contains enough traffic for the stated troubleshooting objective.

The capture is timestamped and archived with an identifiable session name.

# 3. Core Wi-Fi Validation Filters

| Purpose | Wireshark Display Filter | Expected Evidence |
| --- | --- | --- |
| All management frames | wlan.fc.type == 0 | Beacons, probes, authentication, association, action frames as present. |
| All control frames | wlan.fc.type == 1 | ACK, CTS, RTS, Block ACK and related control traffic when present. |
| All data frames | wlan.fc.type == 2 | Data, QoS Data, Null Data and related data traffic when present. |
| Beacons | wlan.fc.type_subtype == 0x08 | Periodic AP beacon frames. |
| Probe Requests | wlan.fc.type_subtype == 0x04 | Client discovery/probing traffic. |
| Probe Responses | wlan.fc.type_subtype == 0x05 | AP responses to probes. |
| EAPOL | eapol | 802.1X / WPA authentication traffic when a client joins. |
| Authentication | wlan.fc.type_subtype == 0x0b | 802.11 authentication exchange when observed. |
| Association Requests | wlan.fc.type_subtype == 0x00 | Client association requests when observed. |
| Association Responses | wlan.fc.type_subtype == 0x01 | AP association responses when observed. |

# 4. Radio Metadata Checks

Radiotap header is present and decodes correctly.

RSSI/signal information is populated on representative frames.

Channel/frequency information is populated on representative frames.

PHY information or duration is available where supported by the capture hardware/driver.

Data-rate information is available where supported by the capture hardware/driver.

The same metadata fields remain consistently populated across the capture rather than appearing only on isolated frames.

# 5. Validated WLAN Pi Go Capture Result

The validated reference capture demonstrated the following engineering-grade characteristics:

Radiotap headers were present and decoded by Wireshark.

Beacon frames were present from multiple BSSIDs/SSIDs.

Probe Requests and Probe Responses were observed.

Control traffic including ACK/CTS/Block ACK was observed.

Data, QoS Data, QoS Null and Null Data traffic was observed.

RSSI values were populated per frame.

Channel values were populated and varied across the observed RF environment.

PHY duration and data-rate fields were visible in representative frames.

The resulting PCAPNG was suitable for wireless engineering analysis.

# 6. Scenario-Specific Validation

The core gate is necessary but not sufficient for every troubleshooting scenario. Generate the traffic required by the test objective.

| Scenario | Evidence to Require |
| --- | --- |
| General RF survey | Beacons, probes, RSSI, channel and PHY metadata. |
| Roaming investigation | Capture a client while moving between APs; look for probe, authentication, association/reassociation and data continuity. |
| WPA/802.1X authentication | Start capture before the client joins; verify authentication/association and EAPOL evidence. |
| Client connectivity | Capture the client’s management exchange and subsequent data/ACK behavior. |
| Voice / real-time traffic | Capture sustained client traffic and validate QoS Data, sequence behavior and control-frame visibility. |
| DFS/channel investigation | Capture on the target channel and verify the expected channel/radio metadata is present. |

# 7. Validation Procedure

1. Open the PCAPNG in Wireshark and confirm the file loads normally.

2. Check the encapsulation and confirm Radiotap/802.11 decoding.

3. Apply the management, control, and data filters and record whether each class is present.

4. Apply the beacon, probe request, and probe response filters for discovery-oriented captures.

5. Check representative frames for RSSI, channel/frequency, PHY duration and data-rate metadata.

6. For authentication or association testing, verify the capture contains the expected event sequence; do not mark absent events as failures when the scenario did not generate them.

7. Record packet count, capture duration, capture source, test objective and any filters used.

8. Assign a final capture grade and archive the PCAPNG with the session documentation.

# 8. Capture Grade

| Grade | Meaning | Disposition |
| --- | --- | --- |
| A / A+ | Required metadata and frame classes are present; capture is suitable for the stated objective. | Use for engineering analysis. |
| B | Capture is readable and useful, but one or more optional metadata/frame classes are missing. | Use with stated limitations. |
| C | File is readable but insufficient for the intended troubleshooting objective. | Recapture with corrected scope/setup. |
| FAIL | Capture is corrupted, not native 802.11/Radiotap, or otherwise unusable. | Do not use for engineering conclusions. |

# 9. Common Misinterpretations

Absence of EAPOL does not mean the capture is broken; EAPOL requires an authentication event to occur during the capture.

Absence of association frames does not mean the monitor interface failed; the client may already have been associated before capture began.

An Ethernet capture of the Mac’s WLAN Pi USB interface is not equivalent to an OTA 802.11 monitor capture.

A capture with many packets is not automatically a good capture. Radio metadata and scenario relevance matter.

The current Wireshark ExtCap problem is an external vendor integration limitation and should not be treated as evidence that the WLAN Pi Go itself cannot capture OTA traffic.

# 10. Final Validation Checklist

☐ PCAPNG opens in Wireshark.

☐ Radiotap/802.11 decoding confirmed.

☐ Management frames evaluated.

☐ Control frames evaluated.

☐ Data frames evaluated.

☐ RSSI verified.

☐ Channel metadata verified.

☐ Scenario-specific evidence verified.

☐ Capture metadata recorded.

☐ Capture grade assigned.

☐ PCAPNG archived with the session.

# 11. Revision History

Version 1.0.0 – Initial capture-validation procedure based on the validated WLAN Pi Go OTA capture workflow and representative Radiotap PCAPNG analysis.