# RF Survey Workflow

Document 13  |  Version 1.0.0

| Purpose | Defines the repeatable WLAN Pi Go RF survey workflow, including collection, analysis, export, and validation of a survey snapshot. |
| --- | --- |
| Audience | Wireless engineers performing onsite RF discovery and engineering assessments. |
| Prerequisites | See the relevant setup and deployment procedures. (Document is an operational reference, not a substitute for those prerequisites.) |
| Status | Knowledge Base document; content reflects the validated project state as of August 2026. |

# 1. Survey Workflow

Start a working session and complete the WLAN Pi Go preflight.

Launch the toolkit and select RF Survey / Quick RF Survey.

Allow the survey process to complete its scan and analysis.

Review the generated survey summary and derived analysis views.

Export the survey package when the results are suitable for retention or review.

Record the survey package identifier and location in the engineering work item.

# 2. Survey Package

A validated survey package contains a metadata record, raw scan output, primary survey output, survey analysis, channel analysis, security analysis, hidden-network analysis, enterprise-network analysis, and neighbor inventory.

| Artifact | Purpose |
| --- | --- |
| metadata.txt | Capture time, host, and survey context. |
| raw_scan.txt | Raw scan evidence retained for troubleshooting. |
| survey.txt | Primary survey results. |
| survey_analysis.txt | Derived survey summary. |
| channel_analysis.txt | Channel distribution and RF observations. |
| security_analysis.txt | Observed WLAN security classifications and MFP status. |
| hidden_networks.txt | Observed hidden-network inventory. |
| enterprise_networks.txt | Enterprise-network inventory. |
| neighbor_inventory.txt | BSSID/SSID neighbor inventory. |

# 3. Validation

Confirm the snapshot timestamp is plausible for the work being performed.

Confirm the BSS inventory is populated and internally consistent across reports.

Confirm 2.4 GHz, 5 GHz, and 6 GHz results are represented when those bands are expected to be in scope.

Treat RSSI as an observation, not a complete RF health conclusion. Correlate channel, bandwidth, utilization, and other evidence before making design decisions.

Validated project example: A completed survey package contained 57 BSSs across 2.4 GHz, 5 GHz, and 6 GHz and included channel, security, hidden-network, enterprise, and neighbor reports.

# 4. Retention and Reporting

Keep the survey package together rather than copying individual text files without their metadata.

Use the exported package as the evidence set for engineering notes.

Do not overwrite a prior survey package when collecting a new site snapshot; retain a unique package directory.

# 5. Troubleshooting

| Symptom | Action |
| --- | --- |
| No survey snapshot | Run the survey workflow again and verify the generated survey directory. |
| Incomplete analysis output | Check the survey package manifest and repeat the analysis from the same snapshot. |
| Unexpected band counts | Confirm the capture hardware and scan coverage before interpreting the RF result. |
| Stale snapshot | Collect a new survey rather than basing a current decision on an old snapshot. |