# Client Profiler

Document 14  |  Version 1.0.0

| Purpose | Documents the WLAN Pi Go client-profiler workflow for validating client association and wireless security behavior. |
| --- | --- |
| Audience | Wireless engineers validating client compatibility, security negotiation, and feature support. |
| Prerequisites | See the relevant setup and deployment procedures. (Document is an operational reference, not a substitute for those prerequisites.) |
| Status | Knowledge Base document; content reflects the validated project state as of August 2026. |

# 1. Available Profiles

| Profile | Purpose |
| --- | --- |
| 802.11be (EHT) WPA3 Profiler | Modern Wi-Fi 7 / WPA3 client validation. |
| 802.11ax (HE) WPA2 Profiler | Wi-Fi 6 / WPA2 client validation. |
| Voice Validation | Voice-oriented association and feature validation. |

# 2. Workflow

Connect the WLAN Pi Go and complete the standard preflight.

Start the toolkit and select the required client-profiler workflow.

Load the profile that matches the engineering test objective.

Associate the test endpoint to the WLAN Pi profiler environment.

Review the generated association/capability information.

Record the endpoint, profile, band, channel, security mode, and notable capabilities in the test notes.

# 3. What to Validate

802.11k / 11v / 11r support where reported by the client.

802.11w Management Frame Protection support.

WPA2/WPA3 and AKM negotiation as applicable to the profile.

Pairwise cipher and authentication method.

11n / 11ac / 11ax / 11be capability indicators when exposed by the client and profile.

Band and channel used during the association.

# 4. Interpreting Results

The profiler is a controlled client-validation tool. Its results should be interpreted as evidence about the tested endpoint and profile, not as proof that every endpoint in an enterprise behaves identically.

Validated project example: A prior profiler test reported 11r, 11k, 11v, 11w, RSNX SAE H2E, GCMP-256, FT-SAE, and 11n/11ac/11ax capability indicators on a 5 GHz test association.

# 5. Troubleshooting

| Issue | Check |
| --- | --- |
| Association fails | Confirm the selected profile, band, security settings, and client capabilities. |
| Expected feature not shown | Confirm the endpoint actually associated to the intended profile and band. |
| Inconsistent results | Repeat with the same profile and record client OS/driver details. |