# Troubleshooting

Document 15  |  Version 1.0.0

| Purpose | Provides a single decision framework for diagnosing common WLAN Pi Go platform, capture, deployment, and documentation problems. |
| --- | --- |
| Audience | Wireless engineers operating or supporting the platform. |
| Prerequisites | See the relevant setup and deployment procedures. (Document is an operational reference, not a substitute for those prerequisites.) |
| Status | Knowledge Base document; content reflects the validated project state as of August 2026. |

# 1. First Response

Confirm the problem is reproducible and note the exact symptom.

Confirm the workstation is using the intended network path and is not unintentionally connected through a different interface.

Run the standard WLAN Pi Go preflight.

Check toolkit version and repository status before changing files.

Capture logs/output before attempting a destructive or configuration-changing workaround.

# 2. Common Conditions

| Condition | Primary checks |
| --- | --- |
| SSH fails | Confirm USB network connectivity, device address, and WLAN Pi Go availability. |
| Toolkit deployment fails | Check SSH, source path, remote directory, and deployment verification output. |
| TERM warning | Interactive terminal environment is not set; do not treat this alone as a toolkit failure. |
| Tar timestamp warnings | Compare workstation and WLAN Pi Go clocks; warnings seen during deployment were non-blocking in the validated deployment. |
| No OTA capture | Verify oscium-capture is active, TCP 6174 is listening, and a supported capture application is connected. |
| MetaGeek login issue | Verify web login, app state, application logs, and current proxy/security state before resetting credentials repeatedly. |
| Wireshark ExtCap hangs | Do not repeatedly reinstall the Go. The vendor reproduced the hang and escalated it to engineering. |

# 3. Capture Troubleshooting Decision Tree

1. Can the Go be reached over USB? If no, resolve workstation connectivity first.

2. Is oscium-capture active and listening on TCP 6174? If no, restore the Go runtime.

3. Does a supported capture application detect the Go? If no, inspect discovery and application configuration.

4. Does the application produce a PCAPNG file with Radiotap/802.11 frames? If no, validate the capture path before interpreting RF behavior.

5. If Wireshark ExtCap alone fails while MetaGeek successfully captures, treat the issue as an ExtCap integration problem rather than a radio/hardware failure.

# 4. Evidence to Retain

Command output showing service state and listening ports.

Toolkit version and Git commit/tag.

Relevant application logs.

Capture file and capture-validation results.

Screenshots showing exact UI errors when required.

Vendor ticket/reference and engineering response.